Using PSPICE and importing it’s data to RAM Commander

- Use CAD sofwtare to create a scheme:

- Create a Net-List, save it to the disk:

* source EXAMPLE

RBIAS         N00078 VDD  20k

RC1         OUT1 VDD  10k

RC2         OUT2 VDD  10k

RS2         0 N00112  1k

RS1         N00061 N00240  1k

Q1         N00078 N00078 VEE Q2N2222

Q2         N00315 N00078 VEE Q2N2222

Q3         OUT1 N00061 N00315 Q2N2222

Q4         OUT2 N00112 N00315 Q2N2222

V2         VDD 0 12V

V3         VEE 0 -12V

C1         OUT1 OUT2  5p

V1         N00240 0  AC 1

+SIN 0 0.1V 5MEG 0 0 0

For example, OrCAD software has a function “Create NetList”, which prepares a file of required format for PSPICE:

If the schema contains components which have additional libraries with PSPICE component models, insert links to this libraries into the PSPICE NetList:

*LIBRARIES:

* LOCAL LIBRARIES :

.LIB "C:\PROGRAM FILES\ORCADLITE\CAPTURE\LIBRARY\PSPICE\EVAL.LIB"

* FROM [PSPICE NETLIST] SECTION OF C:\WINDOWS\PSPICE.INI FILE:

.LIB "C:\PROGRAM FILES\ORCADLITE\CAPTURE\LIBRARY\PSPICE\EVAL.LIB"

Add line with real power supply parameters (minimal and maximal voltage – in this example 3 and 5 volts) and calculation step (in this example 1 volt).

.DC V1 3.0 5.0 1

Add line which will tell to PSPICE that it is an end of the file:

.END

Here is the entire file:

* source EXAMPLE

RBIAS         N00078 VDD  20k

RC1         OUT1 VDD  10k

RC2         OUT2 VDD  10k

RS2         0 N00112  1k

RS1         N00061 N00240  1k

Q1         N00078 N00078 VEE Q2N2222

Q2         N00315 N00078 VEE Q2N2222

Q3         OUT1 N00061 N00315 Q2N2222

Q4         OUT2 N00112 N00315 Q2N2222

V2         VDD 0 12V

V3         VEE 0 -12V

C1         OUT1 OUT2  5p

V1         N00240 0  AC 1

+SIN 0 0.1V 5MEG 0 0 0

*LIBRARIES:

* LOCAL LIBRARIES :

.LIB "C:\PROGRAM FILES\ORCADLITE\CAPTURE\LIBRARY\PSPICE\EVAL.LIB"

* FROM [PSPICE NETLIST] SECTION OF C:\WINDOWS\PSPICE.INI FILE:

.LIB "C:\PROGRAM FILES\ORCADLITE\CAPTURE\LIBRARY\PSPICE\EVAL.LIB"

.DC V1 3.0 5.0 1

.END

- Build RAM Commander’s product tree with components used in the schema. Define calculation methods, resistors and capacitors styles etc. (all the necessary data except stresses, which will come from PSPICE).

- Perform import from PSPICE:

See the Log file – it shows stress calculated stress values:

- See RAM Commander components data – the stress data was imported: