# IMPORTING PARTS FROM EXTERNAL LIBRARIES

RAM Commander provides a utility for importing component data from external libraries or databases. RAM Commander reads the library import file and encodes it into its own library. This saves the laborious task of manually keying an entire component library. If you have a reliability project on another system, use the import utility to convert it into the RAM Commander format.

## Structure of the library import file

The library import file consist of lines—one line for each component. Each line of the library import file includes two mandatory fields:

- Item Code, 4 characters—selected from the table starting on page 2.

- Component Generic Name, 42 characters—left justified arbitrary string. If the actual Generic Name is less than 42 characters, add trailing blanks.

For example, a record for a coil inductor with Generic Name 181714 would appear in the library import file as

CIND181714

In addition, each line includes optional field definition pairs—field names and their actual values. Field definition pairs are separated with semicolons, and assignment is represented by an equals sign. The syntax for a field definition pair is

<Field name>=<Value>;<Field name>=<Value>;...

A complete list of field names for each item code is presented in the section “Values for Selectable”, starting on page 4.

### Data Types

There are two types of data types in a library import file.

Numeric fields (N) Floating or integer numbers, unrestricted field length.

Selectable fields (S) Number associated with a predefined string value selected from the tables below.

None of the fields appearing in the tables below are mandatory. Regardless, you’ll get best results by including  the “Type” field. For example,

ICDI7400             TYPE=1

represents an Gate Logic Array digital IC with generic name 7400.

### Field Name Abbreviations

Field name abbreviations appear in the tables in the section “Values for Selectable Fields”.

##### To build a library import file record

- Identify the component’s item code from the table on page 2.

- Write the component item code to the ASCII file.

- Write a left-justified generic number (up to 42 characters long, add trailing blanks as necessary).

- Locate the required field for the corresponding item code in the tables presented below.

- For each field, do the following:

- If the field is type “N”, write the field name and its associated value to the ASCII file.

- If the field is type “S”, use a value appearing in the tables in the next section.

- Write a semicolon after each field in the record.

Repeat the above procedure for each part you want to import into RAM Commander’s library.

For example, if you have a digital logic IC, with nonhermetic package, complexity 12 gates, and generic name 74HC00, the corresponding record in the library import file would be

### ICDI74HC00                                    TYPE=1; PACK=15; GATE=12;

### Item Code Abbreviations

The following table lists all item code abbreviations. Use the abbreviations when building the import file

| Item Code | Abbreviation |
| --- | --- |
| Capacitor | CAPC |
| Coil, inductive device | CIND |
| Connector and IC sockets | CONR |
| Fuse | FUSE |
| High frequency diode | HFDI |
| High frequency transistor | HFTR |
| IC Analog | ICAN |
| IC Digital | ICDI |
| IC Memory | ICME |
| Low frequency diode | LFDI |
| Low frequency transistor | LFTR |
| Optoelectronic device | OPTE |
| Potentiometer | POTN |
| Printed wiring board | PWBD |
| Quartz Crystal | QCRY |
| Relay (can be tree hierarchy item) | RELY |
| Resistor | RESI |
| Switch | SWIT |
| Bubble memory | BUBM |
| Circuit breaker | CBRK |
| Connection | CONT |
| Filter | FLTR |
| Hybrid IC | HYBR |
| Lamps incandescent | LAMP |
| Laser gas | LGAS |
| Laser Semiconductor device | LSEM |
| Laser solid state | LSOL |
| Meter | METR |
| Miscellaneous | MISC |
| Rotating device | ROTD |
| Surface acoustic wave | SACW |
| Substrate | SUBS |
| Tube | TUBE |

## Values for Selectable Fields

Each item code is characterized by various select fields. The following tables list, for each item code, what the select fields are and their legal values.

### IC memory (ICME)

| Field name | Abbreviation | Field Type |
| --- | --- | --- |
| Memory type | TYPE | S |
| Technology | TECH | S |
| Package | PACK | S |
| Quality | QUAL | S |
| Subtype for EEPROM | ESBT | S |
| Error correction for EEPROM | EECC | S |
| Subtype for SRAM | SRSB | S |
| Number of pins | PINS | N |
| Number of bits [bits] | BITS | N |
| Theta junction to case [C/watt] | TETA | N |
| Temperature junction max [C] | TMAX | N |
| Max Power dissipation [watts] | PMAX | N |
| Maximum supply voltage [volts] | VMAX | N |
| Years of production | PRYR | N |
| Maturity year - begining of production | MTYR | N |
| Manufacturing year | MNYR | N |
| Utilization period [hours] | UTIL | N |
| Evaluation for CNET | EVAL | S |
| Custom screening groups | QLV1 | S |
| Custom screening groups | QLV2 | S |
| Custom screening groups | QL3B | S |
| Custom screening groups | QL3C | S |
| Custom screening groups | QLV4 | S |
| Custom screening groups | QLV5 | S |
| Custom screening groups | QLV6 | S |
| Custom screening groups | QLV7 | S |
| Custom screening groups | QLV8 | S |
| Custom screening groups | QLV9 | S |
| Custom screening groups | QL10 | S |
| Custom screening groups | QL11 | S |
| Screening for CNET | SCRN | S |
| Frequency rated [MHZ] | FREQ | N |
| User-definable PieQ | PIEQ | N |
| Fanout rated | FANR | N |
| Vin rated | VINR | N |
| Mounting | MOUN | S |
| Learning factor | LEAR | S |

Abbreviation: TYPE	Field name: Type of IC Memory

| Type | Value | Type | Value |
| --- | --- | --- | --- |
| ROM | - 1 | DRAM | 8 |
| RAM | - 2 | SRAM | 9 |
| PROM | - 3 | REPROM | 10 |
| UVEPROM | - 4 | OTP | 11 |
| EPROM | - 5 | FLASH | 12 |
| EEPROM | - 6 | FIFO | 13 |
| EAPROM | - 7 |  |  |

Abbreviation: TECH	Field name: IC Memory technology

| Type | Value | Type | Value |
| --- | --- | --- | --- |
| ASTTL |  | MNOS |  |
| CML |  | PMOS |  |
| TTL |  | NMOS |  |
| HTTL |  | HMOS |  |
| FTTL |  | CCD |  |
| DTL |  | CMOS |  |
| ECL |  | HCMOS |  |
| ALSTTL |  | HTCMOS |  |
| LTTL |  | SOS |  |
| STTL |  | MOS |  |
| LSTTL |  | BIPOLAR |  |
| IIL |  | BiCMOS |  |
| IIIL |  | F |  |
| ISL |  |  |  |

Abbreviation: PACK	Field name: Device package

| Type | Value | Type | Value |
| --- | --- | --- | --- |
| HERM_DIP |  | CAN |  |
| NHER_DIP |  | LCC |  |
| GLAS_DIP |  | LCC_N |  |
| HERM_PGA |  | LLCC |  |
| NHER_PGA |  | LLCC_N |  |
| HERM_SMT |  | HERMETIC |  |
| NHER_SMT |  | NON_HERM |  |
| FLATPACK |  |  |  |

Abbreviation: QUAL	Field name: Quality

| Type | Value | Type | Value |
| --- | --- | --- | --- |
| S |  | Enhanced quality | 12 |
| S_1 |  | CECC or equivalent | 13 |
| B |  | Neither CECC, nor equivalent | 14 |
| B_1 |  | 0 | 15 |
| B_2 |  | User-definable | 16 |
| D |  | A2 | 17 |
| COMMER_ UNKNOWN |  | A3 | 17 |
| CUSTOM_ SCREEN |  | A4 | 19 |
| 1 |  | C1 | 20 |
| 2 |  | C2 | 21 |
| 3 |  |  |  |

Abbreviation: ESBT	Field name: Subtype for EEPROM

| Type | Value | Type | Value |
| --- | --- | --- | --- |
| FLOTOX |  | TEXTURED_POLY |  |

Abbreviation: EECC	Field name: Error correction for EEPROM

| Type | Value | Type | Value |
| --- | --- | --- | --- |
| NO_ON_CHIP |  | TWO_NEEDS_ONE |  |
| ON_CHIP_HAM |  |  |  |

Abbreviation: SRSB	Field name: Subtype for SRAM

| Type | Value | Type | Value |
| --- | --- | --- | --- |
| Fast, FIFO |  | Double access |  |
| Low consumption |  |  |  |

Abbreviation: EVAL	Field name: Evaluation for CNET

| Type | Value | Type | Value |
| --- | --- | --- | --- |
| With |  | Without |  |

Abbreviation: QLV1, QLV2,QL3B,QL3C,	Field name: Custom screening groups

QLV4, QLV4, QLV6, QLV7,

QLV8, QLV9, QL10,QL11

| Type | Value | Type | Value |
| --- | --- | --- | --- |
| No |  | Yes |  |

Abbreviation: SCRN	Field name: Screening for CNET

| Type | Value | Type | Value |
| --- | --- | --- | --- |
| With - first year |  | Without - first year |  |
| With - more than one year |  | Without - more than one year | 4 |

Abbreviation: MOUN	Field name: Mounting

| Type | Value | Type | Value |
| --- | --- | --- | --- |
| Through hole | 1 | Surface Mount | 2 |

Abbreviation: LEAR	Field name: Learning Factor

| Type | Value | Type | Value |
| --- | --- | --- | --- |
| Procedure has stabilized | 1 | New device in initial production | 3 |
| Quality controls have not stabilized | 2 |  |  |

### IC analog (ICAN)

| Field name | Abbreviation | Field Type |
| --- | --- | --- |
| Analog type | TYPE | S |
| Technology | TECH | S |
| Package | PACK | S |
| Quality | QUAL | S |
| GaAs application | APPL | S |
| Number of pins | PINS | N |
| Number of transistors | TRNS | N |
| Theta junction to case [C/watt] | TETA | N |
| Temperature junction max [C] | TMAX | N |
| Max Power dissipation [watts] | PMAX | N |
| Maximum supply voltage [volts] | VMAX | N |
| Years of production | PRYR | N |
| Maturity year - begining of production | MTYR | N |
| Manufacturing year | MNYR | N |
| Utilization period [hours] | UTIL | N |
| Evaluation for CNET | EVAL | S |
| Custom screening groups | QLV1 | S |
| Custom screening groups | QLV2 | S |
| Custom screening groups | QL3B | S |
| Custom screening groups | QL3C | S |
| Custom screening groups | QLV4 | S |
| Custom screening groups | QLV5 | S |
| Custom screening groups | QLV6 | S |
| Custom screening groups | QLV7 | S |
| Custom screening groups | QLV8 | S |
| Custom screening groups | QLV9 | S |
| Custom screening groups | QL10 | S |
| Custom screening groups | QL11 | S |
| Screening for CNET | SCRN | S |
| Current rated [A] | IMAX | N |
| User-definable PieQ | PIEQ | N |
| Vin rated | VINR | N |
| Mounting | MOUN | S |
| Learning factor | LEAR | S |

Abbreviation: TYPE	Field name: Analog type

| Type | Value | Type | Value |
| --- | --- | --- | --- |
| Linear |  | Telecomms SLIC | 10 |
| MPROC |  | Telecomms SLAC | 11 |
| GAAS_MMIC |  | Telecomms CODEC | 12 |
| OP_AMP |  | Linear Low Voltage | 13 |
| COMMUTATION |  | Linear High Voltage | 14 |
| Digital / Linear |  | DA/AD Converter | 15 |
| Telephone circuits |  | DSP devices | 16 |
| ISDN Line Interface |  | ASIC-Gate Arrays | 17 |
| ISDN_Data Handling |  | ASIC-Standard Cells | 18 |

Abbreviation: TECH	Field name: Technology

| Type | Value | Type | Value |
| --- | --- | --- | --- |
| ASTTL |  | MNOS |  |
| CML |  | PMOS |  |
| TTL |  | NMOS |  |
| HTTL |  | HMOS |  |
| FTTL |  | CCD |  |
| DTL |  | CMOS |  |
| ECL |  | HCMOS |  |
| ALSTTL |  | HTCMOS |  |
| LTTL |  | SOS |  |
| STTL |  | MOS |  |
| LSTTL |  | BIPOLAR |  |
| IIL |  | BiCMOS |  |
| IIIL |  | F |  |
| ISL |  | JFET | 28 |

Abbreviation: PACK	Field name: Package

| Type | Value | Type | Value |
| --- | --- | --- | --- |
| HERM_DIP |  | CAN |  |
| NHER_DIP |  | LCC |  |
| GLAS_DIP |  | LCC_N |  |
| HERM_PGA |  | LLCC |  |
| NHER_PGA |  | LLCC_N |  |
| HERM_SMT |  | HERMETIC |  |
| NHER_SMT |  | NON_HERM |  |
| FLATPACK |  |  |  |

Abbreviation: QUAL	Field name: Quality

| Type | Value | Type | Value |
| --- | --- | --- | --- |
| S |  | Enhanced quality | 12 |
| S_1 |  | CECC or equivalent | 13 |
| B |  | Neither CECC, nor equivalent | 14 |
| B_1 |  | 0 | 15 |
| B_2 |  | User-definable | 16 |
| D |  | A2 | 17 |
| COMMER_ UNKNOWN |  | A3 | 17 |
| CUSTOM_ SCREEN |  | A4 | 19 |
| 1 |  | C1 | 20 |
| 2 |  | C2 | 21 |
| 3 |  |  |  |

Abbreviation: APPL	Field name: GaAS application

| Type | Value | Type | Value |
| --- | --- | --- | --- |
| Low Noise & Low Power |  | Unknown |  |
| Driver & High power |  |  |  |

Abbreviation: EVAL	Field name: Evaluation for CNET

| Type | Value | Type | Value |
| --- | --- | --- | --- |
| With |  | Without |  |

Abbreviation: QLV1, QLV2,QL3B,QL3C,	Field name: Custom screening groups

QLV4, QLV4, QLV6, QLV7,

QLV8, QLV9, QL10,QL11

| Type | Value | Type | Value |
| --- | --- | --- | --- |
| No |  | Yes |  |

Abbreviation: SCRN	Field name: Screening for CNET

| Type | Value | Type | Value |
| --- | --- | --- | --- |
| With - first year |  | Without - first year |  |
| With - more than one year |  | Without - more than one year | 4 |

Abbreviation: MOUN	Field name: Mounting

| Type | Value | Type | Value |
| --- | --- | --- | --- |
| Through hole | 1 | Surface Mount | 2 |

Abbreviation: LEAR	Field name: Learning Factor

| Type | Value | Type | Value |
| --- | --- | --- | --- |
| Procedure has stabilized | 1 | New device in initial production | 3 |
| Quality controls have not stabilized | 2 |  |  |

### IC digital (ICDI)

| Field name | Abbreviation | Field Type |
| --- | --- | --- |
| Digital type | TYPE | S |
| Technology | TECH | S |
| Package | PACK | S |
| Quality | QUAL | S |
| VHSIC/VLSI Manufacturing Process | VLMP | S |
| Number of pins | PINS | N |
| Number of bits f or MPROC [bits] | BITS | N |
| Number of gates | GATE | N |
| Theta junction to case [C/watt] | TETA | N |
| Temperature junction max [C] | TMAX | N |
| Max Power dissipation [watts] | PMAX | N |
| Maximum supply voltage [volts] | VMAX | N |
| Years of production | PRYR | N |
| Maturity year - begining of production | MTYR | N |
| Manufacturing year | MNYR | N |
| Utilization period [hours] | UTIL | N |
| Total scribed chip Die Area in [cm^2] | VLDA | N |
| Feature Size [microns] | VLFS | N |
| Voltage Ranges which will cause the part to fail [volts] (Vth) | VLVR | N |
| Subtype for GaAs | GSBT | S |
| Evaluation for CNET | EVAL | S |
| Custom screening groups | QLV1 | S |
| Custom screening groups | QLV2 | S |
| Custom screening groups | QL3B | S |
| Custom screening groups | QL3C | S |
| Custom screening groups | QLV4 | S |
| Custom screening groups | QLV5 | S |
| Custom screening groups | QLV6 | S |
| Custom screening groups | QLV7 | S |
| Custom screening groups | QLV8 | S |
| Custom screening groups | QLV9 | S |
| Custom screening groups | QL10 | S |
| Custom screening groups | QL11 | S |
| Screening for CNET | SCRN | S |
| Frequency rated [MHZ] | FREQ | N |
| User-definable PieQ | PIEQ | N |
| Fanout rated | FANR | N |
| Vin rated | VINR | N |
| Mounting | MOUN | S |
| Learning factor | LEAR | S |

Abbreviation: TYPE	Field name: Digital type

| Type | Value | Type | Value |
| --- | --- | --- | --- |
| GATE_LOG_ARR | 1 | INTERFACE | 11 |
| CGA | 2 | Prediffuses | 12 |
| SH_R | 3 | LCA | 13 |
| PLA | 4 | GAL | 14 |
| PAL | 5 | EPLD | 15 |
| MPROC | 6 | MAX | 16 |
| VHSIC_LOGIC | 7 | FPGA anti fusible | 17 |
| VHSIC_GATE | 8 | FPLA | 18 |
| GAAS_DIG | 9 | Gate Array | 19 |
| SEMI_CUSTOM | 10 | Peripheral | 20 |

Abbreviation: TECH	Field name: Technology

| Type | Value | Type | Value |
| --- | --- | --- | --- |
| ASTTL | 1 | MNOS | 15 |
| CML | 2 | PMOS | 16 |
| TTL | 3 | NMOS | 17 |
| HTTL | 4 | HMOS | 18 |
| FTTL | 5 | CCD | 19 |
| DTL | 6 | CMOS | 20 |
| ECL | 7 | HCMOS | 21 |
| ALSTTL | 8 | HTCMOS | 22 |
| LTTL | 9 | SOS | 23 |
| STTL | 10 | MOS | 24 |
| LSTTL | 11 | BIPOLAR | 25 |
| IIL | 12 | BiCMOS | 26 |
| IIIL | 13 | F | 27 |
| ISL | 14 |  |  |

Abbreviation: PACK	Field name: Package

| Type | Value | Type | Value |
| --- | --- | --- | --- |
| HERM_DIP | 1 | CAN | 9 |
| NHER_DIP | 2 | LCC | 10 |
| GLAS_DIP | 3 | LCC_N | 11 |
| HERM_PGA | 4 | LLCC | 12 |
| NHER_PGA | 5 | LLCC_N | 13 |
| HERM_SMT | 6 | HERMETIC | 14 |
| NHER_SMT | 7 | NON_HERM | 15 |
| FLATPACK | 8 |  |  |

Abbreviation: QUAL	Field name: Quality

| Type | Value | Type | Value |
| --- | --- | --- | --- |
| S | 1 | Enhanced quality | 12 |
| S_1_E | 2 | CECC or equivalent | 13 |
| B | 3 | Neither CECC, nor equivalent | 14 |
| B_1 | 4 | 0 | 15 |
| B_2 | 5 | User-definable | 16 |
| D | 6 | A2 | 17 |
| COMMER_ UNKNOWN | 7 | A3 | 17 |
| CUSTOM_SCREEN | 8 | A4 | 19 |
| 1 | 9 | C1 | 20 |
| 2 | 10 | C2 | 21 |
| 3 | 11 |  |  |

Abbreviation: VLMP	Field name: VHSIC/VLSI Manufacturing Process

| Type | Value | Type | Value |
| --- | --- | --- | --- |
| QML_QPL |  | NQML_NQPL |  |

Abbreviation: GSBT	Field name: GaAs Subtype

| Type | Value | Type | Value |
| --- | --- | --- | --- |
| Normally on transistors |  | Normally off transistors |  |

Abbreviation: EVAL	Field name: Evaluation for CNET

| Type | Value | Type | Value |
| --- | --- | --- | --- |
| With |  | Without |  |

Abbreviation: QLV1, QLV2,QL3B,QL3C,	Field name: Custom screening groups

QLV4, QLV4, QLV6, QLV7,

QLV8, QLV9, QL10,QL11

| Type | Value | Type | Value |
| --- | --- | --- | --- |
| No |  | Yes |  |

Abbreviation: SCRN	Field name: Screening for CNET

| Type | Value | Type | Value |
| --- | --- | --- | --- |
| With - first year |  | Without - first year |  |
| With - more than one year |  | Without - more than one year | 4 |

Abbreviation: MOUN	Field name: Mounting

| Type | Value | Type | Value |
| --- | --- | --- | --- |
| Through hole | 1 | Surface Mount | 2 |

Abbreviation: LEAR	Field name: Learning Factor

| Type | Value | Type | Value |
| --- | --- | --- | --- |
| Procedure has stabilized | 1 | New device in initial production | 3 |
| Quality controls have not stabilized | 2 |  |  |

### Bubble memory (BUBM)

| Field name | Abbreviation | Field Type |
| --- | --- | --- |
| Technology | TECH | S |
| Package | PACK | S |
| Quality | QUAL | S |
| # of Bubble Chips | CHIP | N |
| # of Dissipative elem. | DISP | N |
| Number of pins | PINS | N |
| Number of gates | BITS | N |
| Number of bits | GATE | N |
| Years of production | PRYR | N |
| Maturity year - begining of production | MTYR | N |
| Manufacturing year | MNYR | N |
| Manuf.max rated data rate | MANR | N |
| Theta junction to case [C/watt] | TETA | N |
| Temperature junction max [C] | TMAX | N |
| Max Power dissipation [watts] | PMAX | N |
| User-definable PieQ | PIEQ | N |
| Custom screening groups | QLV1 | S |
| Custom screening groups | QLV2 | S |
| Custom screening groups | QL3B | S |
| Custom screening groups | QL3C | S |
| Custom screening groups | QLV4 | S |
| Custom screening groups | QLV5 | S |
| Custom screening groups | QLV6 | S |
| Custom screening groups | QLV7 | S |
| Custom screening groups | QLV8 | S |
| Custom screening groups | QLV9 | S |
| Custom screening groups | QL10 | S |
| Custom screening groups | QL11 | S |

Abbreviation: TECH	Field name: Technology

| Type | Value | Type | Value |
| --- | --- | --- | --- |
| TTL |  | IIL | 8 |
| HTTL |  | MNOS | 9 |
| DTL |  | PMOS | 10 |
| ECL |  | NMOS | 11 |
| LTTL |  | CCD | 12 |
| STTL |  | CMOS | 13 |
| LSTTL |  | CMOS-SOS | 14 |

Abbreviation: PACK	Field name: Package

| Type | Value | Type | Value |
| --- | --- | --- | --- |
| HERM_DIP |  | CAN |  |
| NHER_DIP |  | LCC |  |
| GLAS_DIP |  | LCC_N |  |
| HERM_PGA |  | LLCC |  |
| NHER_PGA |  | LLCC_N |  |
| HERM_SMT |  | HERMETIC |  |
| NHER_SMT |  | NON_HERM |  |
| FLATPACK |  |  |  |

Abbreviation: QUAL	Field name: Quality

| Type | Value | Type | Value |
| --- | --- | --- | --- |
| S |  | D | 6 |
| S_1 |  | COMMER_ UNKNOWN | 7 |
| B |  | CUSTOM_SCREEN | 8 |
| B_1 |  | User-definable | 9 |
| B_2 |  |  |  |

Abbreviation: QLV1, QLV2,QL3B,QL3C,	Field name: Custom screening groups

QLV4, QLV4, QLV6, QLV7,

QLV8, QLV9, QL10,QL11

| Type | Value | Type | Value |
| --- | --- | --- | --- |
| No |  | Yes |  |

### Resistors (RESI)

| Field name | Abbreviation | Field Type |
| --- | --- | --- |
| Type(style) of resistor | TYPE | S |
| Quality | QUAL | S |
| Number of film resistors (for Network) | NETR | N |
| Characteristic | RESC | S |
| Suffix for resistance | RESS | S |
| Resistance | RESI | N |
| Rated wattage | PMAX | N |
| t1 temperature1 | RET1 | N |
| s1 stress1 | RES1 | N |
| t2 temperature2 | RET2 | N |
| s2 stress2 | RES2 | N |
| Rated voltage | VMAX | N |
| Evaluation for CNET | EVAL | S |
| User-definable PieQ | PIEQ | N |
| Peak rated [W] | PEAK | N |
| Max rated temperature | CAPT | S |

Abbreviation: TYPE	Field name: Style of resistor

| Type | Value | Type | Value | Type | Value |
| --- | --- | --- | --- | --- | --- |
| RC |  | RW23 | 37 | RTH08 | 73 |
| RCR |  | RW24 | 38 | RTH10 | 74 |
| RZ |  | RW29 | 39 | RTH12 | 75 |
| RBR |  | RW30 | 40 | RTH14 | 76 |
| RB |  | RW31 | 41 | RTH16 | 77 |
| RD |  | RW32 | 42 | RTH18 | 78 |
| RE |  | RW33 | 43 | RTH20 | 79 |
| RE60 |  | RW34 | 44 | RTH22 | 80 |
| RE65 |  | RW35 | 45 | RTH24 | 81 |
| RE70 |  | RW36 | 46 | RTH26 | 82 |
| RE75 |  | RW37 | 47 | RTH28 | 83 |
| RE77 |  | RW38 | 48 | RTH30 | 84 |
| RE80 |  | RW39 | 49 | RTH32 | 85 |
| RER |  | RW47 | 50 | RTH34 | 86 |
| RER40 |  | RW55 | 51 | RTH36 | 87 |
| RER45 |  | RW56 | 52 | RTH38 | 88 |
| RER50 |  | RW67 | 53 | RTH39 | 89 |
| RER60 |  | RW68 | 54 | RTH40 | 90 |
| RER65 |  | RW69 | 55 | RTH42 | 91 |
| RER70 |  | RW70 | 56 | RNC | 92 |
| RER75 |  | RW74 | 57 | RNN | 93 |
| RL |  | RW78 | 58 | Thermistor Bead | 94 |
| RLR |  | RW79 | 59 | Thermistor Disk | 95 |
| RN |  | RW80 | 60 | Thermistor Rod | 96 |
| RNR |  | RW81 | 61 | Chip | 97 |
| RW |  | RWR | 62 | Thick/Thin network | 98 |
| RW10 |  | RWR71 | 63 | Carbon Film | 99 |
| RW11 |  | RWR74 | 64 | Metal Oxide & Metal Glaze | 100 |
| RW12 |  | RWR78 | 65 | Metal Film | 101 |
| RW13 |  | RWR80 | 66 | Vitreous Enamel WW | 102 |
| RW14 |  | RWR81 | 67 | Low dissip.- SMT | 103 |
| RW15 |  | RWR82 | 68 | Metal Glaze | 104 |
| RW16 |  | RWR84 | 69 | Metal Oxide Varistor | 105 |
| RW20 |  | RWR89 | 70 | Varistors | 106 |
| RW21 |  | RTH | 71 | Thermistor PPTC Device | 107 |
| RW22 |  | RTH06 | 72 | Pressure Sensitive | 108 |

Abbreviation: QUAL	Field name: Quality of resistor

| Type | Value | Type | Value |
| --- | --- | --- | --- |
| S |  | CECC or equivalent | 11 |
| R |  | Neither CECC, nor equivalent | 12 |
| P |  | 0 | 13 |
| M |  | User-definable | 14 |
| MIL |  | A1l | 15 |
| LOWER |  | A1w | 16 |
| 1 |  | A2 | 17 |
| 2 |  | B1 | 18 |
| 3 |  | B2 | 19 |
| Enhanced quality |  | C | 20 |

Abbreviation: RESC	Field name: Resistor characteristic

| Type | Value | Type | Value |
| --- | --- | --- | --- |
| G |  | Inductively Wound |  |
| N |  | Noninductively Wound |  |

Abbreviation: RESS	Field name: Suffix for resistance

| Type | Value | Type | Value |
| --- | --- | --- | --- |
| OHM |  | MOHM |  |
| KHOM |  |  |  |

Abbreviation: EVAL	Field name: Evaluation for CNET

| Type | Value | Type | Value |
| --- | --- | --- | --- |
| With |  | Without |  |

Abbreviation: CAPT	Field name: Maximum rated temparature

| Type | Value | Type | Value |
| --- | --- | --- | --- |
| 70 | 1 | 150 | 4 |
| 125 | 2 | 155 | 5 |
| 130 | 3 | 200 | 6 |

### Potentiometers (POTN)

| Field name | Abbreviation | Field Type |
| --- | --- | --- |
| Type(style) of potentiometer | TYPE | S |
| Quality | QUAL | S |
| Construction RR | POTR | S |
| Construction RP | POTP | S |
| Number of potentiometer Taps | POTT | N |
| Suffix for resistance | RESS | S |
| Resistance | RESI | N |
| Rated wattage | PMAX | N |
| Rated voltage | VMAX | N |
| t1 temperature1 | RET1 | N |
| s1 stress1 | RES1 | N |
| t2 temperature2 | RET2 | N |
| s2 stress2 | RES2 | N |
| Evaluation for CNET | EVAL | S |
| User-definable PieQ | PIEQ | N |
| Peak rated [W] | PEAK | N |
| Part Kind | KIND | S |

Abbreviation: TYPE	Field name: Type of potentiometer

| Type | Value | Type | Value |
| --- | --- | --- | --- |
| RA |  | Low Precision Carbon NW |  |
| RK |  | Trimmer NW |  |
| RJ |  | High power Enclosed WW |  |
| RJR |  | Lead screw WW |  |
| RP |  | Wire Wound Preset |  |
| RP07 |  | Carbon Preset |  |
| RP11 |  | Cermet Preset |  |
| RP16 |  | Non Preset Wire Wound |  |
| RQ |  | Non Preset Carbon |  |
| RR |  | Non Preset Cermet |  |
| RT |  | Non Prese Conductive Plastic |  |
| RTR |  | Non wirewound cermet (RS) |  |
| RV |  | Non wirewound cermet (RSR) |  |
| RVC |  | Glass Glaze | 28 |

Abbreviation: POTR	Field name: RR Construction

| Type | Value | Type | Value |
| --- | --- | --- | --- |
| RR09A1A9J1031 |  | RR09A4A9J1034 |  |
| RR09A2A9J1032 |  | RR09A5A9J1035 |  |
| RR09A3A9J1033 |  | RR09A6A9J103 |  |

Abbreviation: POTP	Field name: RP Construction

| Type | Value | Type | Value |
| --- | --- | --- | --- |
| Unenclosed |  | Enclosed |  |

Abbreviation: QUAL	Field name: Quality of potentiometer

| Type | Value | Type | Value |
| --- | --- | --- | --- |
| S |  | CECC or equivalent | 11 |
| R |  | Neither CECC, nor equivalent | 12 |
| P |  | 0 | 13 |
| M |  | User-definable | 14 |
| MIL |  | A1 | 15 |
| LOWER |  | A2 | 16 |
| 1 |  | B1 | 17 |
| 2 |  | B2 | 18 |
| 3 |  | C | 19 |
| Enhanced quality |  |  |  |

Abbreviation: RESS	Field name: Suffix for resistance

| Type | Value | Type | Value |
| --- | --- | --- | --- |
| OHM |  | MOHM |  |
| KHOM |  |  |  |

Abbreviation: EVAL	Field name: Evaluation for CNET

| Type | Value | Type | Value |
| --- | --- | --- | --- |
| With |  | Without |  |

Abbreviation: KIND	Field name: Part Kind

| Type | Value | Type | Value |
| --- | --- | --- | --- |
| Fine Tuning | 1 | Switch | 3 |
| Low Power | 2 | General | 4 |

### Capacitors (CAPC)

| Field name | Abbreviation | Field Type |
| --- | --- | --- |
| Type(style) of capacitor | TYPE | S |
| Quality | QUAL | S |
| Constraction | CAPC | S |
| Configuration | CAPF | S |
| Max rated temperature | CAPT | S |
| Suffix for capacitance | CAPS | S |
| Capacitance | CAPI | N |
| Resistance for CSR style | RESI | N |
| Rated voltage | VMAX | N |
| Evaluation for CNET | EVAL | S |
| User-definable PieQ | PIEQ | N |
| Case for CM,CMR | CSCM | S |
| Case for CSR | CCSR | S |

Abbreviation: TYPE	Field name: Type of capacitor

| Type | Value | Type | Value |
| --- | --- | --- | --- |
| CM |  | CT | 28 |
| CMR |  | PC | 29 |
| CB |  | CG | 30 |
| CK |  | CG Fixed | 31 |
| CKR |  | CG Variable | 32 |
| CC |  | CZR | 33 |
| CCR |  | Plastic Fixed | 34 |
| CDR |  | Aluminum Axial Lead Fixed | 35 |
| CSR |  | Aluminum Chassis Mounted Fixed | 36 |
| CL |  | Metallised Polyester & Polycarbonate | 37 |
| CLR |  | Polyester & Polycarbonate Foil | 38 |
| CU |  | Polypropylene | 39 |
| CUR |  | Polystyrene Foil | 40 |
| CE |  | Electrolytic Solid Tantalum (non metal case) | 41 |
| CP |  | Electrolytic Tantalum Foil | 42 |
| CZ |  | Variable | 43 |
| CA |  | Synthetic Film | 44 |
| CPV |  | Tantalum Wet | 45 |
| CH |  | CWR - Chip, Electrolytic, Tantalum | 46 |
| CHR |  | CRL - Electrolytic, Nonsolid, Tantalum Cathode | 47 |
| CQ |  | Poly Metal Film | 48 |
| CQR |  | Electrolytic Aluminum Wet | 49 |
| CFR |  | Metallised Polycarbonate | 50 |
| CRH |  | Polycarbonate Foil | 51 |
| CY |  | Terylene film | 52 |
| CYR |  | Polytetrafluoroethylene film | 53 |
| CV |  |  |  |

Abbreviation: CAPT	Field name: Maximum rated temparature

| Type | Value | Type | Value |
| --- | --- | --- | --- |
| T65 |  | T125 |  |
| T70 |  | T150 |  |
| T85 |  | T170 |  |
| T100 |  | T175 |  |
| T105 |  | T200 |  |

Abbreviation: CAPC	Field name: Capacitor construction

| Type | Value | Type | Value |
| --- | --- | --- | --- |
| SLUG_(CLR79) |  | FOIL_NON_HERMETIC |  |
| FOIL_HERMETIC |  | SLUG_NON_HERM |  |
| SLUG_HERMETIC |  |  |  |

Abbreviation: CAPF	Field name: Capicitor configuration

| Type | Value | Type | Value |
| --- | --- | --- | --- |
| FIXED |  | VARIABLE |  |

Abbreviation: QUAL	Field name: Capicitor quality

| Type | Value | Type | Value |
| --- | --- | --- | --- |
| D |  | 3 | 16 |
| C |  | Enhanced quality | 17 |
| S |  | CECC or equivalent | 18 |
| B |  | Neither CECC, nor equivalent | 19 |
| T |  | 0 | 20 |
| R |  | User-definable | 21 |
| P |  | A1b | 22 |
| M |  | A1q | 23 |
| MIL |  | A1l | 24 |
| L |  | A1w | 25 |
| LOWER |  | A2 | 26 |
| CM_DIPP |  | B1 | 27 |
| CM_MOLD |  | B2 | 28 |
| 1 |  | C1 | 29 |
| 2 |  | C2 | 30 |

Abbreviation: CAPS	Field name: Suffix for capacitance

| Type | Value | Type | Value |
| --- | --- | --- | --- |
| uF |  | pF |  |
| nF |  |  |  |

Abbreviation: EVAL	Field name: Evaluation for CNET

| Type | Value | Type | Value |
| --- | --- | --- | --- |
| With |  | Without |  |

Abbreviation: CSCM	Field name: Case for CM,CMR

| Type | Value | Type | Value |
| --- | --- | --- | --- |
| Molded |  | Dipped |  |

Abbreviation: CCSR	Field name: Case for CSR

| Type | Value | Type | Value |
| --- | --- | --- | --- |
| Metallic case |  | Dipped |  |
| Molded case - chip |  |  |  |

### Switch (SWIT)

| Field name | Abbreviation | Field Type |
| --- | --- | --- |
| Switches type | TYPE | S |
| Quality | QUAL | S |
| Contact form | FCON | S |
| Evaluation for CNET | EVAL | S |
| # of active contacts | NCON | N |
| # of keys (touches) | KEYS | N |
| Current rated [A] | IMAX | N |
| Power rated [W] | PMAX | N |
| Voltage rated [V] | VMAX | N |
| User-definable PieQ | PIEQ | N |

Abbreviation: TYPE	Field name: Type of switch

| Type | Value | Type | Value |
| --- | --- | --- | --- |
| Toggle or Pushbutton |  | Thumbwheel Switches | 18 |
| Push Button |  | Screw | 19 |
| Pushbutton Snap-action |  | Dil Slider | 20 |
| Pushbutton Nonsnap-action |  | Dil Toggle | 21 |
| Toggle Standard |  | Keypad | 22 |
| Toggle Miniature |  | Elast Keypad | 23 |
| Toggle Micro |  | Press Keypad | 24 |
| Toggle Snap-action |  | Keyboard | 25 |
| Toggle Nonsnap-action |  | Centrifugal | 26 |
| Toggle Pushbutton, Rocker, Slide |  | Limit | 27 |
| Rocker or Slide |  | Liquid Level | 28 |
| Sensitive |  | Microwave | 29 |
| Sensitive With Actuation Differential > 0.002 inch |  | Pressure | 30 |
| Sensitive With Actuation Differential < 0.002 inch |  | Reed | 31 |
| Rotary |  | Thermal | 32 |
| Rotary RF Ceramic |  | Inching | 33 |
| Rotary Medium Power |  |  |  |

Abbreviation: FCON	Field name: Contact form

| Type | Value | Type | Value |
| --- | --- | --- | --- |
| SPST |  | DPDT |  |
| DTST |  | 3PDT |  |
| SPDT |  | 4PDT |  |
| 3PST |  | 6PDT |  |
| 4PST |  |  |  |

Abbreviation: QUAL	Field name: Quality of diode

| Type | Value | Type | Value |
| --- | --- | --- | --- |
| MIL-SPEC |  | CECC or equivalent | 9 |
| ER |  | Neither CECC, nor equivalent | 10 |
| LOWER |  | User-definable | 11 |
| 0 |  | A | 12 |
| 1 |  | B1 | 13 |
| 2 |  | B2 | 14 |
| 3 |  | C | 15 |
| Enhanced quality |  |  |  |

Abbreviation: EVAL	Field name: Evaluation for CNET

| Type | Value | Type | Value |
| --- | --- | --- | --- |
| With |  | Without |  |

### Relay (RELY)

| Field name | Abbreviation | Field Type |
| --- | --- | --- |
| Relays type | TYPE | S |
| Quality | QUAL | S |
| Contact form | FCON | S |
| Temperature Rating | CAPT | S |
| Type of protection | PROT | S |
| Evaluation for CNET | EVAL | S |
| # of breaker active contacts | CINT | S |
| # of inverser active contacts | CINV | S |
| Current for CNET | COUR | S |
| Package for NONOP | PACK | S |
| Current rated [A] | IMAX | N |
| Voltage rated [V] | VMAX | N |
| User-definable PieQ | PIEQ | N |
| Learning factor | LEAR | S |

Abbreviation: TYPE	Field name: Relay type

| Type | Value | Type | Value |
| --- | --- | --- | --- |
| General Purpose |  | High Voltage Vacuum (Ceramic) |  |
| Reed |  | Medium Power Armature |  |
| Mercury |  | Medium Power Mercury Wetted |  |
| Meter, movement |  | Medium Power Magnetic Latching |  |
| Dry Armature |  | Medium Power Mechanical Latch |  |
| Dry Reed |  | Medium Power Balanced Armature |  |
| Dry Reed Miniature |  | Medium Power Solenoid |  |
| Dry Reed Mercury |  | Contactor |  |
| Dry Mercury Wetted |  | Contactor Armature |  |
| Dry Magnetic Latching |  | Contactor Mechanical Latching |  |
| Dry Balanced Armature |  | Contactor Balanced Armature |  |
| Dry Solenoid |  | Contactor Solenoid |  |
| General Purpose Armature |  | Solid State Relay |  |
| General Purpose Balanced Armature |  | Hybrid TDR |  |
| General Purpose Solenoid |  | Solid State TDR |  |
| Sensitive Armature |  | Mercury wetted reed low power/Balanced |  |
| Sensitive Mercury Wetted |  | Mercury wetted reed low power/Latching balanced |  |
| Sensitive Magnetic Latching |  | Dry reed /Balanced |  |
| Sensitive Meter Movement |  | Dry reed /Latching balanced |  |
| Sensitive Balanced Armature |  | Electromechanical |  |
| Polarized Armature |  | Electromechanical-Miniature or card |  |
| Polarized Meter Movement |  | Electromechanical-European type |  |
| Vibrating Dry Reed |  | Industrial |  |
| Vibrating Mercury Wetted |  | High voltage vacuum |  |
| High Speed Armature |  | Power mercury wetted |  |
| High Dry Reed |  | Galvanometriques |  |
| Thermal Time Delay Bimetal |  | Diaphragm |  |
| Electronic Time Delay |  | Low Profile |  |
| Latching |  | BT Type 23 |  |
| Latching Magnetic Reed |  | BT Types 47,51 and 53 |  |
| Latching Mercury Wetted |  | BT Types 3000 |  |
| Latching Balanced Armature |  | BT Types 19 |  |
| High Voltage Vacuum (Glass) |  | Printed wired board (DIL) | 66 |

Abbreviation: FCON	Field name: Contact form

| Type | Value | Type | Value |
| --- | --- | --- | --- |
| SPST |  | DPDT |  |
| DTST |  | 3PDT |  |
| SPDT |  | 4PDT |  |
| 3PST |  | 6PDT |  |
| 4PST |  |  |  |

Abbreviation: CINT	Field name: # of breaker active contacts

| Type | Value | Type | Value |
| --- | --- | --- | --- |
| No contacts |  | 3 |  |
| 1 |  | 4 |  |
| 2 |  |  |  |

Abbreviation: CINV	Field name: # of inverser active contacts

| Type | Value | Type | Value |
| --- | --- | --- | --- |
| No contacts |  | 3 |  |
| 1 |  | 4 |  |
| 2 |  | 6 |  |

Abbreviation: CAPT	Field name: Temperature rating

| Type | Value | Type | Value |
| --- | --- | --- | --- |
| 40°C |  | 100°C |  |
| 70°C |  | 125°C |  |
| 85°C |  |  |  |

Abbreviation: PROT	Field name: Protection type

| Type | Value | Type | Value |
| --- | --- | --- | --- |
| Mercury wetted |  | Dust protected |  |
| Hermetically sealed |  | Exposed contacts |  |
| Sealed |  |  |  |

Abbreviation: QUAL	Field name: Quality of relay

| Type | Value | Type | Value |
| --- | --- | --- | --- |
| R |  | 2 | 13 |
| P |  | 3 | 14 |
| X |  | Enhanced quality | 15 |
| U |  | CECC or equivalent | 16 |
| M |  | Neither CECC, nor equivalent | 17 |
| L |  | User-definable | 18 |
| ER |  | A1 | 19 |
| NON_ER |  | A2 | 20 |
| MIL-SPEC |  | B1 | 21 |
| LOWER |  | B2 | 22 |
| 0 |  | C | 23 |
| 1 |  |  |  |

Abbreviation: EVAL	Field name: Evaluation for CNET

| Type | Value | Type | Value |
| --- | --- | --- | --- |
| With |  | Without |  |

Abbreviation: COUR	Field name: Current for CNET

| Type | Value | Type | Value |
| --- | --- | --- | --- |
| DC |  | AC |  |

Abbreviation: PACK	Field name: Package

| Type | Value | Type | Value |
| --- | --- | --- | --- |
| Non-hermetic |  | Hermetic |  |

Abbreviation: LEAR	Field name: Learning Factor

| Type | Value | Type | Value |
| --- | --- | --- | --- |
| Products by produced stably | 1 | Development and new products | 3 |
| Quality of products is not stable | 2 |  |  |

### Connector (CONR)

| Field name | Abberviation | Field Type |
| --- | --- | --- |
| Configuration | TYPE | S |
| Quality | QUAL | S |
| Contact size (Jauge) | CSZE | S |
| Cat.clim. | CAPT | S |
| Material | MATR | S |
| Evaluation for CNET | EVAL | S |
| # of active contacts | NCON | N |
| Current rated [A] | IMAX | N |
| User-definable PieQ | PIEQ | N |
| Vin rated [V] | VINR | N |
| Socket type | SOCK | S |
| Jack construction | JACK | S |

Abbreviation: TYPE	Field name: Connector type - Configuration

| Type | Value | Type | Value |
| --- | --- | --- | --- |
| General |  | IC sockets | 12 |
| Rack/Panel |  | Tin/Tin-Lead-plated | 13 |
| Circular |  | Metal-plated | 14 |
| Power |  | Multipin | 15 |
| Coaxial Press |  | Link | 16 |
| Coaxial Lock |  | Hexagonal | 17 |
| Coaxial/rf |  | Rectangular | 18 |
| Triaxial/rf |  | Telephone | 19 |
| Print Wiring Board |  | Coaxial, Electric | 20 |
| PCB one-piece |  | Coaxial, Optical | 21 |
| PCB two-piece |  | Ribbon Cable | 22 |

Abbreviation: MATR	Field name: Connector material

| Type | Value | Type | Value |
| --- | --- | --- | --- |
| A |  | Gold/Gold |  |
| B |  | Silver/Silver |  |
| C |  | Tin/Tin |  |
| D |  | Others |  |

Abbreviation: CSZE	Field name: Contact size

| Type | Value | Type | Value |
| --- | --- | --- | --- |
| 12 GA |  | 18 GA | 6 |
| 16 GA |  | 24 GA | 7 |
| 20 GA |  | 28 GA | 8 |
| 22 GA |  | 30 GA | 9 |
| 26 GA |  | 32 GA | 10 |

Abbreviation: QUAL	Field name: Quality

| Type | Value | Type | Value |
| --- | --- | --- | --- |
| MIL-SPEC |  | Neither CECC, nor equivalent | 9 |
| LOWER |  | User-definable | 10 |
| 0 |  | A1 | 11 |
| 1 |  | A2 | 12 |
| 2 |  | B1 | 13 |
| 3 |  | B2 | 14 |
| Enhanced quality |  | C | 15 |
| CECC or equivalent |  |  |  |

Abbreviation: EVAL	Field name: Evaluation for CNET

| Type | Value | Type | Value |
| --- | --- | --- | --- |
| With |  | Without |  |

Abbreviation: CAPT	Field name: Temperature rating

| Type | Value | Type | Value |
| --- | --- | --- | --- |
| 85°C |  | 200°C | 5 |
| 125°C |  | 250°C | 6 |
| 150°C |  | 175°C | 7 |
| 185°C |  |  |  |

Abbreviation: SOCK	Field name: Socket type

| Type | Value | Type | Value |
| --- | --- | --- | --- |
| DIP |  | Relay | 5 |
| SIP |  | Transistor | 6 |
| Chip Carrier |  | El. Tube, CRT | 7 |
| PGA |  |  |  |

Abbreviation: JACK	Field name: Jack construction

| Type | Value | Type | Value |
| --- | --- | --- | --- |
| Reed | 1 | Linear reed | 3 |
| Pinhole | 2 |  |  |