# RBD export structure

Sample RBD:

RBD data can be written to the text file, for example:

| Line N | Contents |
| --- | --- |
| 1 | 10 |
| 2 | 14 |
| 3 | 0 0 |
| 4 | 0.000000 0.250000 5 |
| 5 | 0.800000 |
| 6 | 17 |
| 7 | 100 |
| 8 | 3 –1 1 4.9999999255 0.0000000000 1.00 -1 0. 0. 0.000000 Device1 |
| 9 | 5 –1 1 3.3333332009 0.0000000000 1.00 -1 0. 0. 0.000000 Device2 |
| 10 | 9 –1 1 2.4999999627 0.0000000000 1.00 -1 0. 0. 0.000000 Device3 |
| 11 | 12 -1 1 2.4999999627 0.0000000000 1.00 -1 0. 0. 0.000000 Device3 |
| 12 | 15 -1 1 2.4999999627 0.0000000000 1.00 -1 0. 0. 0.000000 Device3 |
| 13 | 10000 -1 1 2.0000000000 0.0000000000 1.00 -1 0. 0. 0.000000 Device4 |
| 14 | 10001 -1 1 2.0000000000 0.0000000000 1.00 -1 0. 0. 0.000000 Device4 |
| 15 | 10002 -1 1 2.0000000000 0.0000000000 1.00 -1 0. 0. 0.000000 Device4 |
| 16 | 10003 -1 1 2.0000000000 0.0000000000 1.00 -1 0. 0. 0.000000 Device4 |
| 17 | 10004 -1 1 2.0000000000 0.0000000000 1.00 -1 0. 0. 0.000000 Device4 |
| 18 | 1 1 2 1 1 4 3 5 7 19 |
| 19 | 7 1 1 1 1 3 9 12 15 |
| 20 | 19 1 3 3 5 5 10000 10001 10002 10003 10004 |

Description of file structure:

Line 1 – number of elements in diagram

Line 3-7 – service fields

Lines 8-17 – diagram elements data, space-delimited:

| Position | Data |
| --- | --- |
| 1 | ID |
| 2 | Always –1 |
| 3 | FR distribution code (Exponential, Weibull, Normal, etc.) |
| 4 | FR Distribution parameter 1 |
| 5 | FR Distribution parameter 2 |
| 6 | Load |
| 7 | Repair distribution code (Exponential, Weibull, Normal, etc.), if no repair than value –1 |
| 8 | Repair distribution parameter 1 |
| 9 | Repair distribution parameter 2 |
| 10 | Inspection time |
| 11 | Reference designator |

Lines count is equal to number in Line 1.

Lines 18-20 – description of elements connection. Each line represents element and a group of its sons - similarily connected elements.

| Position | Data |
| --- | --- |
| 1 | ID |
| 2 | Always 1 |
| 3 | Group element’s (son’s) connection type:2-serial, 1-parallel, 3-K-out-of-N |
| 4 | If K-out-of-N – K, else 1 |
| 5 | If K-out-of-N – N, else 1 |
| 6 | Number of serial elements in this group |
| 7-… | List of ID’s of elements (number of ID’s is in position N6 |

For example:

Line 18 in example file is: 1 1 2 1 1 4  3 5 7 19

That means that elements 3,5,7,19 are connected serially. Elements  3 and 5 defined in lines 8 and 9.

Element 7 is defined in line 19: 7 1 1 1 1 3  9 12 15

(in first position of line 19 is “7” – element ID).

In position number 3 is 1 – sign of parallel connection. There are 3 parallel elements – 9, 12 and 15, defined in lines 9-12 of file. Block of these parallel elements is connected to elements 3 and 5 with a serial connection.

Element 19 is defined in line 20: 19 1 3 3 5 5 10000 10001 10002 10003 10004

It is a block of K-out-of-N elements with ID 10000, 10001, 10002, 10003, 10004, where K=3.