Explanation of UTEC Mission Profile dialog box columns

The UTEC Mission Profile dialog box is located in the Project Configuration screen. To open the Project Configuration screen, choose Configuration from the Project menu. In the Project Configuration screen, select UTEC MP to display the Mission Profile dialog box.

You will see a list of profiles with their data is displayed.

To better understand the data columns explained below, please refer to the UTE C 80-810 document, page 17, “Mission Profile-Parameters necessary to define the mission profile”.

EXPLANATION OF MISSION PROFILE DATA COLUMNS

Part I. Working Phases

| Fields | Explanation |
| --- | --- |
| Time1, Time2, Time3, Time4, Time5 | annual ratio of times for the PCB, corresponds to 1 - i in the UTEC document |
| Temp1,Temp2, Temp3, Temp4, Temp5 | average ambient temperature of the PCB, corresponds to (ac)1 - (ac) i in the UTEC document |

Part II. Thermal Cycling

| Fields | Explanation |
| --- | --- |
| Cycl1, Cycl2, Cycl3, Cycl4, Cycl5 | annual number of thermal cycles, corresponds to ni in the UTEC document |
| dT1, dT2, dT3, dT4, dT5 | swing of the thermal variation, corresponds to ΔTi in the UTEC document |
| Stor1, Stor2, Stor3, Stor4, Stor5 | parameter of corresponding phase i (1 to 5) - corresponds to On or Off for On/Off phase or to Storage or Permanent Working for storage or permanent working phase, refer to UTE C 80-810 document |